Evaluation Report on Remote Sensing Methodology

- Introduction

This report presents an evaluation of the use of remote sensing methods for identifying and detecting land use in the Al Baten region. The primary objective is to assess the reliability of this approach by comparing satellite data with ground surveys.

- Methodology

  - Processing and Analysis of Satellite Images

The analysis of satellite images was conducted using various indices. The NDWI was employed to identify water bodies, particularly for detecting irrigation basins. Additionally, the GNDVI and NDVI indices were used to evaluate vegetation cover and classify different categories of vegetation.

  - Overlaying Results with Ground Surveys

The results obtained from the satellite image analysis were overlaid with ground surveys provided by the ecology team. This step was followed by a comparative analysis to assess the concordance between satellite data and field observations.

  - Random Sampling

A random sampling of 105 points was conducted in areas identified as irrigation basins, as well as in locations where vegetation was detected. Samples were also collected from areas where no detection was made by satellite imagery but were identified by ecologists in the field.

  - Field Validation and Data Analysis

Tableau 1: Distribution of Validation Points

| Remote sensing/field | Pool irrigation | Fileds | Some olive trees | Ziziphus | Abandoned | House | Fence | spreading | Nu | Shep farming | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pool irrigation | 14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 14 |
| Vegetation | 0 | 43 | 0 | 5 | 0 | 0 | 0 | 12 | 0 | 15 | 75 |
| Bare Soil | 0 | 0 | 7 | 0 | 2 | 2 | 2 | 0 | 3 | 0 | 16 |
| Total | 14 | 43 | 7 | 5 | 2 | 2 | 2 | 12 | 3 | 15 | 105 |

- Irrigation Basins: The detected points correspond exactly to the irrigation basins, indicating a precision of 100% for this category.

- Vegetation: A total of 75 points were classified as vegetation, including 43 points for fields and 12 points for spreading. However, 15 points showing sheep droppings were incorrectly classified as vegetation, revealing confusion in this category.

- Bare Soil: The 16 points classified as bare soil include errors, notably 7 points identified as some olive trees and other elements (houses, fences). This highlights the difficulty in distinguishing between these surfaces.

Tableau 2: Confusion Matrix

| Remote sensing/field | Pool irrigation | Vegetation | Bare Soil | Total | Accuracy |
| --- | --- | --- | --- | --- | --- |
| Pool irrigation | 14 | 0 | 0 | 14 | 100 |
| Vegetation | 0 | 60 | 15 | 75 | 80 |
| Bare Soil | 0 | 0 | 16 | 16 | 100 |
| Total | 14 | 60 | 31 | 105 |  |

Kappa Index (κ)

The Kappa index measures the agreement between satellite observations (remote sensing method) and ground surveys.

Kappa=Po​-Pe​1-pe

Where:

- 𝑃𝑜 is the observed agreement proportion.

- 𝑃𝑒 is the expected agreement proportion by chance.

Step 1: Calculate Observed Agreement Proportion (𝑃𝑜)

Po​=Sum of correct diagonal values​Total points= 14+60+16105= 90105≈0,857

Step 2: Calculate Expected Agreement Proportion (𝑃𝑒)

- For expected compliant points:

Pe=∑Row Total×Column Total​Total points2= 14×14+75×60+(16×31)1052= 519211025≈0.471

Step 3: Substitute into the Formula

κ=0,857-0,.471 1-0,471 =0,3860,529=0,730

Thus, Kappa Index (κ): 0.730 (73.0%), confirming a good level of agreement between satellite observations and ground surveys, despite some discrepancies noted for grazing areas.

- Conclusion and Perspectives

The analysis of remote sensing demonstrated a good correspondence with ground surveys, particularly for identifying water-filled irrigation basins using the NDWI index. The Kappa index (κ) of 0.730 (73.0%) indicates a substantial level of agreement between satellite data and field observations. However, challenges remain in detecting unfilled basins and distinguishing between vegetation and sheep droppings. Additionally, while bare soil is easily identifiable, some fenced bare areas remain undetectable due to the resolution of 10 meters.

To enhance the accuracy of future classifications, it is essential to refine analytical methods and explore higher resolutions. This will facilitate better differentiation of various terrain characteristics and optimize the use of remote sensing data.

| Characteristic | Identification | Difficulties | Necessary Improvements |
| --- | --- | --- | --- |
| Irrigation Basins | Detectable when filled with NDWI | Visual detection for unfilled basins | Improve detection methods |
| Vegetation | Possible identification with NDVI/GNDVI | Confusion with sheep droppings | Refine classification to avoid confusion |
| Bare Soil | Easily identifiable | Fenced bare areas undetectable | - |

This conclusion highlights both the strengths and limitations of remote sensing analysis while proposing avenues for improvement to optimize future outcomes.